# 22 kW Solar Pump Inverter Procurement List

For a conventional 3-phase 380 V asynchronous water pump in Dandong, Liaoning

Purpose. Compare 13 solar pump VFDs that can directly drive an existing 22 kW / 45 A class pump. The first quote round should prioritize POWTRAN, CLV, FRECON, INVT, and VEICHI, using the same technical request so quotes are directly comparable.

Price note. Prices below are public listings, historical export values, or negotiation targets captured for comparison. They are not binding domestic quotations. Ask every supplier for the tax-included delivered price to Dandong, warranty term, stock status, cold-weather operating range, and repair location.

# Recommended RFQ Specification

Request: 22 kW solar pump inverter for direct drive of an existing conventional 3-phase 380 V asynchronous water pump; rated output current at least 45 A; maximum PV DC input around 800 V; preferred MPPT operating point about 500-600 V; solar direct drive; preferably 3-phase 380 V grid backup; installation in Dandong, Liaoning. Quote one unit, tax included and delivered, with warranty, low-temperature range, stock status, and repair location.

# Priority Summary

| Priority | Model | Published / Reference Price | Electrical Fit |
| --- | --- | --- | --- |
| 1 | POWTRAN PI550A1-S 022G3 | US$188-285 ¦ RMB 1,260-1,910 | Excellent |
| 2 | FRECON PV500-4T-022B-H | US$532 ¦ about RMB 3,560 | Verify PV voltage |
| 3 | INVT GD100-022G-4-PV | US$501 export benchmark ¦ about RMB 3,350; domestic quote required | Excellent |
| 4 | CLV PV500-0220G3 | Quote required; target RMB 2,000-3,000 | Excellent |

# Detailed Procurement List

## 1. POWTRAN PI550A1-S 022G3

Manufacturer: Dalian Powtran  |  Price: US$188-285 | RMB 1,260-1,910
Key specifications: 22 kW; 45 A; 3-phase 380 V; PV 250-800 V; MPPT  |  Warranty: 18 months
Contact: 0411-84821133 | 400-0411-755 | inverter@powtran.com  |  Buying note: Best first quote for Northeast China. Ask for the exact solar-pump PI550A1-S version, a 22 kW / 45 A / G3 nameplate photo, tax-included delivery to Dandong, and the local warranty procedure.

Purchase / inquiry: Open product or inquiry page

## 2. FRECON PV500-4T-022B-H

Manufacturer: FRECON Electric  |  Price: US$532 | about RMB 3,560
Key specifications: 22 kW; 45 A; PV 230-800 V; PV array about 28.6 kW; recommended PV 600-750 V  |  Warranty: Confirm
Contact: +86-755-88605930 | overseas@frecon.com.cn  |  Buying note: Strong quality and support structure. The recommended PV range is above a 500-600 V target, so send the exact panel Voc and Vmp for string-design confirmation.

Purchase / inquiry: Open product or inquiry page

## 3. INVT GD100-022G-4-PV

Manufacturer: INVT  |  Price: US$501 export benchmark | about RMB 3,350; domestic quote required
Key specifications: 22 kW; 45 A; 380 V; max PV 800 V; MPPT point about 550 V; PV/grid hybrid  |  Warranty: Confirm
Contact: 400-700-9997 | invt@invt.com  |  Buying note: Best established-brand option and an excellent electrical fit. Use INVT's authorized-dealer verification before payment; avoid generic 'GD100-PV compatible' listings.

Purchase / inquiry: Open product or inquiry page

## 4. CLV PV500-0220G3

Manufacturer: Shenyang CLV Drives  |  Price: Quote required; target RMB 2,000-3,000
Key specifications: 22 kW; 45 A; 380 V; PV max 800 V; MPPT 450-600 V; recommended input 540 V  |  Warranty: Confirm
Contact: Manager Xiao: 136 9217 0201 | sales@clvdrives.com | support@clvdrives.com  |  Buying note: Closest match to a 500-600 V array and very convenient for Dandong. Ask whether 22 kW stock and repair service are available in Shenyang.

Purchase / inquiry: Open product or inquiry page

## 5. VEICHI SI23-D5-022G-A

Manufacturer: Suzhou VEICHI  |  Price: US$653 historical export value | about RMB 4,375; domestic quote required
Key specifications: 22 kW; 45 A; PV 250-780 V; 3-phase 380-460 V; AM and PM motor support  |  Warranty: Confirm
Contact: 0512-66171988 | 400-600-0303  |  Buying note: Mature industrial product with a documented 22 kW conventional-pump case. Confirm the PV string design before ordering.

Purchase / inquiry: Open product or inquiry page

## 6. M-Driver PV500-0220G3

Manufacturer: Shenzhen M-Driver  |  Price: US$338-375 | RMB 2,265-2,515
Key specifications: 22 kW; 45 A; 380 V; MPPT; direct asynchronous-motor drive  |  Warranty: 18 months
Contact: +86-755-89369291 | +86 13651498092 | admin@m-driver.com | support@m-driver.com  |  Buying note: Good low-cost option. Ask in writing who pays return freight if it fails and whether replacement control boards and power boards are stocked.

Purchase / inquiry: Open product or inquiry page

## 7. ZK SU100-D3-022G-B

Manufacturer: Shenzhen ZK Electric  |  Price: Quote required; target RMB 2,000-3,000
Key specifications: 22 kW; 45 A; PV max 800 V; 0-380/440 V; Vmp/Voc about 540/648 V  |  Warranty: 18 months
Contact: +86-13725501611 | frank@zundrive.com  |  Buying note: Good voltage fit and a published -20 C to +60 C operating range. For outdoor use, specify the IP65 version or a heated, weatherproof enclosure.

Purchase / inquiry: Open product or inquiry page

## 8. ZK SG600-022GB-4T

Manufacturer: ZK / ZUN Drive  |  Price: US$250-420 | RMB 1,675-2,815
Key specifications: 22 kW / 30 HP; 45 A; 380/440 V; MPPT; PV and grid AC input  |  Warranty: 18 months
Contact: +86-13725501611 | frank@zundrive.com  |  Buying note: One of the lowest-cost credible alternatives. Suitable as a price benchmark, but weaker Northeast service than Powtran, CLV, FRECON, or INVT.

Purchase / inquiry: Open product or inquiry page

## 9. CKMINE SP800-022G-4

Manufacturer: Zhejiang Kaimin / CKMINE  |  Price: US$355-390 | RMB 2,380-2,610
Key specifications: 22 kW; 380-440 V; 45 A claimed; MPPT about 99%; 0-400 Hz  |  Warranty: 18 months
Contact: +86-18989731667  |  Buying note: Before paying, require the actual 22 kW nameplate and PDF datasheet. Earlier materials showed inconsistent output-current values.

Purchase / inquiry: Open product or inquiry page

## 10. IDEEI / Lixiang LX100-022G-4-PV

Manufacturer: Haiyan Lixiang  |  Price: US$216 prior sample listing | about RMB 1,450; refresh quote
Key specifications: 22 kW; 45 A; AC 380 V; PV 250-800 V; 0-400 Hz; MPPT; RS485  |  Warranty: Confirm
Contact: Official product inquiry page (no direct phone or email was published in the prior listing)  |  Buying note: Very low initial cost. Buy one unit first and test under full pump load for MPPT stability, thermal performance, and protection behavior before any multi-unit purchase.

Purchase / inquiry: Open product or inquiry page

## 11. ZK500-P-4T-022G-B

Manufacturer: ZK  |  Price: Quote required; target below RMB 2,500-3,000
Key specifications: 22 kW; 45 A; PV 250-800 V; recommended DC about 650 V; 0-380 V  |  Warranty: Confirm
Contact: +86-13725501611 | frank@zundrive.com  |  Buying note: Technically suitable, but its recommended 650 V point is above the 500-600 V target. Require panel-string confirmation and competitive delivered pricing.

Purchase / inquiry: Open product or inquiry page

## 12. Raynen RV35-T3-22GB

Manufacturer: Fujian Raynen  |  Price: Quote required
Key specifications: 22 kW; 45/60 A depending on configuration; 0-380/460 V; Vmp/Voc about 540/660 V  |  Warranty: Confirm
Contact: 400-0898-188 | 137-6468-5533  |  Buying note: Technically credible with favorable voltage characteristics. It may move higher if a Northeast distributor can provide clear service coverage and a competitive quote.

Purchase / inquiry: Open product or inquiry page

## 13. FMZ H880G-22RG/30RP-T4

Manufacturer: Shanghai FMZ Industry  |  Price: China quote required; Indonesia listing IDR 26,280,000 reference only
Key specifications: 22 kW; 45/60 A; 380 V; PV max 800 V; MPPT range 300-750 V; PV/AC switching  |  Warranty: Confirm
Contact: +86 15921436186 | +86-515-88685088 | info@fmz-industry.com  |  Buying note: Technically capable, but choose it only if its delivered China price is materially below better-supported products and service terms are acceptable.

Purchase / inquiry: Open product or inquiry page

# Supplier Contact Starting Points

| Supplier | Contact | Recommended Question |
| --- | --- | --- |
| POWTRAN | 0411-84821133 ¦ 400-0411-755 ¦ inverter@powtran.com | Can you quote PI550A1-S 022G3 delivered to Dandong and handle warranty in Liaoning? |
| CLV Drives | Manager Xiao: 136 9217 0201 ¦ sales@clvdrives.com | Is PV500-0220G3 22 kW in Shenyang stock, and where is the nearest repair point? |
| FRECON | +86-755-88605930 ¦ overseas@frecon.com.cn | Can this model work with my panel Vmp/Voc and is there a Shenyang channel? |
| INVT | 400-700-9997 ¦ invt@invt.com | Who is the authorized Northeast dealer for GD100-022G-4-PV? |
| VEICHI | 0512-66171988 ¦ 400-600-0303 | Please confirm panel-string design, warranty, and Dandong repair process. |

# Minimum Checks Before Payment

• Exact model and real nameplate photo: 22 kW, at least 45 A, 3-phase 380 V, solar-pump MPPT version.

• Panel-string calculation against actual Voc and Vmp at low temperature, not only the inverter maximum DC rating.

• Tax-included delivered price to Dandong, stock lead time, warranty period, and written repair or replacement process.

• Outdoor installation plan: IP65 machine or a correctly sized weatherproof and heated cabinet for winter conditions.

• For low-cost alternatives, buy one unit first and run a full-load acceptance test before a larger purchase.